Digital Assignment

Title of the Mini Project: Analog Wind Speed Indicator using LM324, Arduino Uno, LEDs, and Relay

Student Details:

Roll No: 24BEC1361

Name of the Student: Zaahir Ali Shaik

Year/Semester: II Year / III Semester Section: B.TECH (ECE)

Course Code & Course Title: Analog Circuits

Academic Year: 2025–2026

Department Name: Electronics and Communication Engineering

Institution Name: VIT Chennai

### Abstract

This project presents a simple and cost-effective Analog Wind Speed Indicator using an LM324 operational amplifier, Arduino Uno, LEDs, and a relay module. The system measures wind speed using a small DC motor that acts as a generator. The voltage generated by the motor is proportional to the wind speed and is amplified by the LM324 op-amp before being read by the Arduino. The Arduino processes the analog signal, converts it into a wind speed value, and triggers visual indicators (LEDs) and a relay based on a preset cutoff speed. This project demonstrates basic interfacing between analog and digital components and provides an introduction to real-world sensor measurement and control applications.

### Objectives

• To design a low-cost and efficient analog wind speed indicator. • To interface LM324 with Arduino for signal amplification and processing. • To implement wind-speed-based control using LEDs and a relay output.

### Problem Statement

Most professional anemometers are expensive and complex. This project aims to develop a simple analog wind speed measurement system using easily available components that can visually and electronically indicate wind speed variations.

### Literature Survey

Several low-cost anemometers use Hall sensors or ultrasonic sensors, but they are expensive or complex to interface. Using a DC motor as a generator provides a linear voltage proportional to wind speed. The LM324 op-amp is a reliable and low-power amplifier suitable for single-supply operation. Arduino Uno offers built-in ADC functionality and digital control, making it ideal for signal measurement and display tasks.

### Tools and Technologies Used

- Arduino Uno

- LM324 Operational Amplifier

- DC Toy Motor (as wind sensor)

- Relay Module (5V)

- LEDs (Red & Green)

- Resistors (100 Ω)

- Capacitor (1 µF)

- Breadboard and jumper wires

- Arduino IDE software

### Proposed System / Methodology

- The toy motor generates voltage proportional to wind speed.

- The LM324 op-amp amplifies this voltage.

- The amplified signal is fed into the Arduino analog input.

- The Arduino converts voltage into wind speed (km/h) using calibration.

- When wind speed exceeds a preset cutoff, the red LED and relay are activated.

- When the speed is below the cutoff, the green LED is ON.

### Implementation

- The DC motor output is connected to the LM324 in a non-inverting amplifier configuration.

- LM324 output is fed into Arduino analog pin A0.

- Green LED is connected to D2, Red LED to D7, and the relay module to D4.

- Arduino code reads the analog voltage, calculates wind speed, and controls the LED and relay states based on a 1 km/h cutoff.

- The serial monitor displays real-time voltage and wind speed.

### Results and Analysis

- The circuit successfully measures small voltage changes produced by wind and displays corresponding wind speed values.

- The system shows accurate LED indications and relay switching around the cutoff speed.

- Even small air movement triggers measurable voltage variations, confirming system sensitivity.

| Parameter | Observed Behavior |
| --- | --- |
| No wind | Voltage ~0V, Green LED ON |
| Low wind (<1 km/h) | Small voltage, Green LED ON |
| Above cutoff (>1 km/h) | Voltage increases, Red LED and Relay ON |

### Conclusion

The Analog Wind Speed Indicator successfully demonstrates how analog sensing, signal amplification, and digital control can be integrated into a single system. Using simple components like an LM324 op-amp and Arduino Uno, real-time environmental measurements can be achieved cost-effectively. This project builds fundamental knowledge applicable to weather monitoring, automation, and control systems.

### Future Enhancement

• Display wind speed on an LCD or OLED display. • Add wireless data transmission (Bluetooth/Wi-Fi). • Use a calibrated anemometer cup for higher accuracy. • Implement data logging to SD card for long-term analysis.

### References

- Arduino.cc Documentation

- Texas Instruments LM324 Datasheet

- TutorialsPoint and CircuitDigest Articles on Anemometer Circuits

- Embedded Systems Design using Arduino – NPTEL Notes

### Annexure (Optional)

- Arduino program code

- Circuit diagram

- Prototype setup photos